# Battleship Puzzle - Technical Breakdown

Version 43 | Hi-Orbit Escape Room

## PAGE 1: How It Works

### Overview

The Battleship puzzle is an 8×8 LED/button matrix game where players must recreate ship patterns displayed on an external NeoPixel display. Players press buttons to toggle LEDs on the matrix, matching the target pattern to win each round.

### Hardware Components

| Component | Purpose | Pins |
| --- | --- | --- |
| 8×8 LED Matrix | Displays player selections & patterns | Rows: A10, A14, 2-7 / Cols: 49-35 (odd) |
| 8×8 Button Matrix | Player input | Rows: A9, A13, 8-13 / Cols: 28, 26, 24, 22, 21, 18-20 |
| NeoPixel Display | Shows target patterns (via Serial) | TX Pin 32 |
| Activation Pin | COGS trigger to start puzzle | Pin 48 (INPUT_PULLUP) |
| Minor Completion | Round win signal (3 pulses) | Pin 52 |
| Major Completion | Full game win signal (1 pulse) | Pin 50 |

### Game Flow

┌─────────────────────────────────────────────────────────────────┐
│  1. STARTUP                                                      │
│     └─> Wait for activation pin (48) to go LOW                  │
│     └─> Play startup animation                                   │
│     └─> Begin cycling unsolved patterns on NeoPixel              │
├─────────────────────────────────────────────────────────────────┤
│  2. PATTERN CYCLING (every 3-5 seconds)                         │
│     └─> Display Pattern A, B, or C on NeoPixel                  │
│     └─> Player sees which "ship" they need to place             │
├─────────────────────────────────────────────────────────────────┤
│  3. PLAYER INPUT                                                 │
│     └─> Player presses button on 8×8 matrix                     │
│     └─> If button is part of ANY unsolved pattern → LED ON      │
│     └─> If button is NOT part of any pattern → ERROR BLINK      │
├─────────────────────────────────────────────────────────────────┤
│  4. PATTERN LOCK                                                 │
│     └─> First valid button press LOCKS that specific pattern    │
│     └─> NeoPixel freezes on locked pattern                      │
│     └─> Only buttons for THAT pattern are now valid             │
├─────────────────────────────────────────────────────────────────┤
│  5. PATTERN COMPLETION                                           │
│     └─> All LEDs match the locked pattern → ROUND WIN           │
│     └─> Blink pattern 4x, send 3 pulses on pin 52               │
│     └─> Pattern marked complete, board clears                   │
├─────────────────────────────────────────────────────────────────┤
│  6. ERROR HANDLING                                               │
│     └─> 3 bad moves → Round reset (6 pulses on pin 52)          │
│     └─> Board clears but completed patterns stay done           │
├─────────────────────────────────────────────────────────────────┤
│  7. GAME WIN                                                     │
│     └─> All 3 patterns complete → 1 pulse on pin 50             │
│     └─> Full board blink animation, game resets                 │
└─────────────────────────────────────────────────────────────────┘

### Pattern Data Structure

Patterns are stored as 8-byte bitmaps (inverted during init):

// Example: Pattern 1 - each byte = 1 row, each bit = 1 column
// 0 = LED ON (ship position), 1 = LED OFF
const unsigned char epd_bitmap_pattern1[8] = { 
    0x1f, 0xff, 0xff, 0xe7, 0xe7, 0xfd, 0xfd, 0xff 
};

### NeoPixel Commands (Serial @ 9600 baud)

| Command | Action |
| --- | --- |
| 'A' | Display Pattern 1 |
| 'B' | Display Pattern 2 |
| 'C' | Display Pattern 3 |
| '1' | Light segment 1 (pattern 1 complete) |
| '2' | Light segment 2 (pattern 2 complete) |
| '3' | Light segment 3 (pattern 3 complete) |
| '9' | Clear all NeoPixels |
| 'Z' | Clear matrix |
| 's' | Startup animation |

## PAGE 2: What Can Be Tweaked

### 🎯 Difficulty & Timing

| Parameter | Location | Default | Description |
| --- | --- | --- | --- |
| interval | Line 63 | 3000 | Pattern cycle time (ms) before switching |
| Last pattern bonus | Line 123 | 5000 | Extra time on last unsolved pattern |
| badMoves threshold | Line 315, 337 | 3 | Errors before round reset |
| Debounce delay | Line 569 | 100 | Button debounce (ms) |

// Make patterns cycle faster (more pressure)
unsigned long interval = 2000;  // Was 3000

// Allow more mistakes before reset
if (badMoves >= 5) {  // Was 3

### 🎨 Animations & Visual Feedback

| Parameter | Location | Default | Effect |
| --- | --- | --- | --- |
| Startup repetitions | Line 89 | 5 | Number of animation waves |
| Startup initial delay | Line 90 | 150 | Animation speed (ms) |
| Acceleration factor | Line 91 | 0.155 | How much each wave speeds up |
| Error blink speed | Line 179 | 200 | Error LED toggle rate (ms) |
| Error blink count | Line 184 | 6 | Toggles (3 blinks = 6 toggles) |
| Pattern win blink | Line 394, 404 | 4 loops, 120ms | Celebration blink |
| Final hold time | Line 417 | 1000 | Pattern stays lit after win |

// Faster error feedback
if (currentTime - lastBlinkTime >= 100) {  // Was 200

// Longer celebration
smartDelay(2000);  // Was 1000

### 🚢 Ship Patterns

Edit the bitmap arrays in initPatterns() (lines 363-377):

// Pattern format: 8 bytes, each byte = 1 row
// Bit = 1 means LED OFF, Bit = 0 means LED ON (inverted later)
const unsigned char init_pattern1[8] = { 
    0x1f,  // Row 0: ___OOOOO (O = ship)
    0xff,  // Row 1: ________ (all off)
    0xff,  // Row 2: ________
    0xe7,  // Row 3: OOO__OOO
    0xe7,  // Row 4: OOO__OOO
    0xfd,  // Row 5: OOOOOO_O
    0xfd,  // Row 6: OOOOOO_O
    0xff   // Row 7: ________
};

Tip: Use an online bitmap editor, export as hex, then paste here.

### 📡 COGS Signal Timing

| Signal | Function | Location | Default |
| --- | --- | --- | --- |
| Minor pulse width | Round/error signals | Line 260, 268, 280 | 100ms |
| Minor pulse gap | Between pulses | Line 271, 284 | 200ms |
| Major pulse width | Game win | Line 259 | 100ms |

// Longer pulses for more reliable detection
digitalWrite(pin, HIGH);
delay(200);  // Was 100
digitalWrite(pin, LOW);

### 🔧 Pin Assignments

// Main control pins (lines 45-47)
#define minorCompletionPin 52  // Round win / error signals
#define majorCompletionPin 50  // Full game win
#define activationPin 48       // COGS start trigger

// LED matrix rows (line 40)
const int ledRows[8] = { A10, A14, 2, 3, 4, 5, 6, 7 };

// LED matrix columns (line 41)  
const int ledCols[8] = { 49, 47, 45, 43, 41, 39, 37, 35 };

// Button matrix rows (line 42)
const int buttonRows[8] = { A9, A13, 8, 9, 10, 11, 12, 13 };

// Button matrix columns (line 43)
const int buttonCols[8] = { 28, 26, 24, 22, 21, 18, 19, 20 };

// NeoPixel serial TX (line 57)
SoftwareSerial neopixel1(-1, 32);  // Change 32 to new TX pin

### 🐛 Debug Mode

// Line 4 - Toggle serial debug output
#define ENABLE_SERIAL_OUTPUT 1  // Set to 0 for production (stops LED flicker)

Note: Serial output causes LED flickering because UART is blocking. Disable for live shows.

### 🎮 Game Mode

// Line 59 - Switch between pattern sets
bool gameSet = false;  // false = Patterns 1-3, true = Patterns 4-6

// Or change via function (saves to EEPROM)
setGameSet(true);  // Switch to patterns 4-6

### ⏱️ Watchdog Timer

// Line 134 - Auto-reset if code hangs
wdt_enable(WDTO_8S);  // Options: WDTO_1S, WDTO_2S, WDTO_4S, WDTO_8S

### Quick Reference: Common Adjustments

| Want To… | Change This |
| --- | --- |
| Make puzzle easier | Increase badMoves threshold (3→5) |
| Make puzzle harder | Decrease interval (3000→2000) |
| Speed up animations | Decrease initialDelay (150→100) |
| Change ship layouts | Edit init_pattern arrays |
| Adjust COGS signals | Modify pulse widths/counts |
| Stop LED flicker | Set ENABLE_SERIAL_OUTPUT to 0 |
| Change trigger pin | Update activationPin define |

Document generated from BattleshipPuzzleV43_thomasFix_110225