# Laser Maze Puzzle - Technical Breakdown

Version 52 | Hi-Orbit Escape Room

## PAGE 1: How It Works

### Overview

The Laser Maze is a complex chain-based light routing puzzle with 25 nodes, 9 rotatable encoder mirrors, and multiple NeoPixel strips. Players must rotate physical mirrors to correct angles, creating a “chain” of light from start to finish. The puzzle has two modes (A and B) that alternate after each win.

### Hardware Components

| Component | Purpose | Pins |
| --- | --- | --- |
| NeoPixel Strip 1 | Encoder nodes 1-9 (9 LEDs) | Pin 4 |
| NeoPixel Strip 2 | Static nodes 19-25 (73 LEDs) | Pin 2 |
| NeoPixel Strip 3 | Static nodes 10-18 (121 LEDs) | Pin 3 |
| 9 AS5600 Encoders | Magnetic rotation sensors | I2C via TCA9548A muxes |
| 26 Laser Transistors | Control laser beams | Various GPIO pins |
| Activation Pin | Start puzzle signal | A15 |
| Force Win Pin | Override to win | A14 |
| COGS Output 1 | Win signal (300ms blip) | A13 |
| COGS Output 2 | Node event pulses | Pin 16 (TX2) |

### Node Layout

┌─────────────────────────────────┐
                    │        LASER MAZE NODES         │
                    ├─────────────────────────────────┤
  ENCODERS (1-9)    │  Strip 1 - Pin 4 - Rotatable   │
  ──────────────────┤  Players rotate these mirrors   │
                    │  Green = correct angle          │
                    │  Red = wrong angle              │
                    ├─────────────────────────────────┤
  STATIC (10-18)    │  Strip 3 - Pin 3 - Fixed       │
  ──────────────────┤  Light up when chain reaches    │
                    │  them, showing path progress    │
                    ├─────────────────────────────────┤
  STATIC (19-25)    │  Strip 2 - Pin 2 - Fixed       │
  ──────────────────┤  Final nodes to complete chain  │
                    └─────────────────────────────────┘

### Chain Logic (Mode A Example)

The puzzle works like a domino chain. Each step requires the previous encoder to be correct:

START: Node 10 always active (rainbow, two lasers)
       ↓
Node 11 + Encoder 1 activate
       ↓ (if Encoder 1 at correct angle)
Node 12 + Encoder 2 activate
       ↓ (if Encoder 2 at correct angle)
Node 13 + Encoder 3 activate
       ↓ ... continues through all encoders ...
       ↓ (if Encoder 9 at correct angle)
Node 23 + Node 25 activate
       ↓
WIN! → COGS pulse, rainbow celebration, mode switches to B

### Target Angles

| Encoder | Mode A Angle | Mode B Angle |
| --- | --- | --- |
| 1 | 61.00° | 354.00° |
| 2 | 331.00° | 309.00° |
| 3 | 90.00° | 116.00° |
| 4 | 280.00° | 280.00° |
| 5 | 187.00° | 183.00° |
| 6 | 6.00° | 359.00° |
| 7 | 288.00° | 34.00° |
| 8 | 318.00° | 145.00° |
| 9 | 163.00° | 303.00° |

Angle Threshold: ±20° (configurable)

## PAGE 2: What Can Be Tweaked

### ⚡ Quick Settings Reference

| Setting | Location | Current Value | Purpose |
| --- | --- | --- | --- |
| angleThreshold | Line 330 | 20.0 | Degrees of tolerance |
| animationInterval | Line 320 | 50ms | Chain update speed |
| LED Brightness | Lines 820-828 | 64 | NeoPixel brightness (0-255) |

### 🎯 Adjusting Angle Tolerance

// Line 330: How close is "close enough"
float angleThreshold = 20.0;  // ±20 degrees

- Easier puzzle: Increase to 30-40

- Harder puzzle: Decrease to 10-15

- Note: Below 10° may be frustrating due to encoder precision

### 🔄 Changing Target Angles

// Lines 111-112: Mode A and B expected angles
float expectedAnglesA[] = { 61.00, 331.00, 90.00, 280.00, 187.00, 6.00, 288.00, 318.00, 163.00 };
float expectedAnglesB[] = { 354.00, 309.00, 116.00, 280.00, 183.00, 359.00, 34.00, 145.00, 303.00 };

To recalibrate: Use the debug command E# (e.g., E1 for encoder 1) via Serial Monitor to see live angle readings, then update the array.

### 💡 LED Color Settings

Encoder LEDs show status with colors:

// In activateEncoderNodeA/B functions
uint32_t encColor = correct ? pixels.Color(0, 255, 0)   // GREEN = correct
                            : pixels.Color(255, 0, 0);  // RED = wrong

// Static nodes when activated
pixels.Color(0, 60, 0)  // Dim green (20% brightness)

To change colors: Modify RGB values in pixels.Color(R, G, B)

### 🔊 COGS Pulse Signaling

The puzzle sends pulses to COGS for events:

// Lines 493-502: Pulse counts for events
void signalStaticNodeActivated() {
  startPulseSequence(COGS_OUTPUT_PIN_2, 3);  // 3 pulses = node ON
}

void signalStaticNodeDeactivated() {
  startPulseSequence(COGS_OUTPUT_PIN_2, 4);  // 4 pulses = node OFF
}

Pulse timing (Line 435-436):

pulser.pulseMs = 100;   // HIGH duration
pulser.gapMs = 200;     // LOW duration between pulses

### 🎮 Debug Commands (Serial Monitor)

| Command | Action |
| --- | --- |
| L# | Flash LED # on Strip 1 |
| M# | Flash LED # on Strip 2 |
| P# | Flash LED # on Strip 3 |
| E# | Live poll Encoder # (1-9) |
| T# | Pulse Transistor # |
| N# | Flash logical Node # (1-25) |
| S | Sequential sweep all nodes |
| C | Test COGS pulse output |

### 🗺️ Node-to-LED Mapping

If physical LEDs don’t match logical nodes, edit mapNodeToLedIndex():

// Lines 375-402: Node → Physical LED index
case 10: return 30;   // Node 10 = LED 31 on Strip 3
case 11: return 22;   // Node 11 = LED 23 on Strip 3
// ... etc

### ⚙️ Chain Modification

To change the puzzle path, edit the chain arrays:

// Lines 114-117: Chain sequences
int chainA[] = { 11, 1, 12, 13, 2, 14, 3, 15, 4, 16, 5, 17, 6, 18, 7, 19, 8, 22, 9, 23 };
int chainB[] = { 12, 2, 13, 14, 6, 15, 1, 16, 4, 17, 3, 24, 5, 25, 8, 20, 7, 22, 9 };

Format: Numbers 1-9 are encoders, 10+ are static nodes

### 🐛 Troubleshooting

| Problem | Solution |
| --- | --- |
| Encoder not reading | Use E# command to check; may need I2C address fix |
| LEDs on wrong strip | Check getNodeStrip() function and mapNodeToLedIndex() |
| Chain stops unexpectedly | Use Serial Monitor to see which encoder is failing |
| Win doesn’t trigger | Check COGS_OUTPUT_PIN and pulse timing |
| Mode doesn’t switch | Verify gameMode variable toggles in chainComplete() |

### 📚 Required Libraries

- Wire.h - I2C communication

- Adafruit_NeoPixel.h - LED strip control

- PeripheralFunctions.h - Custom encoder/mux classes (included)

### 📋 Code Structure

Main Components:
├── setup()                    - Initialize hardware, run tests
├── loop()                     - Main game loop, debug handling
├── updateChain()              - Process chain logic
├── updateChainModeAGraph()    - Mode A specific chain
├── updateChainModeBGraph()    - Mode B specific chain
├── chainComplete()            - Win celebration, mode switch
├── runDebugCommand()          - Serial debug interface
└── Encoder/Mux classes        - I2C sensor handling