# Payphone Puzzle - Technical Breakdown

Version 7 | Hi-Orbit Escape Room

## PAGE 1: How It Works

### Overview

The Payphone puzzle is a code entry puzzle disguised as a vintage payphone. Players must enter a 5-digit code by pressing illuminated buttons. The puzzle features a hall effect sensor (handset pickup detection), an external RGB ring for status feedback, and a secondary white LED strip for atmospheric lighting. When the correct code is entered, the puzzle triggers a celebration and signals completion.

### Hardware Components

| Component | Purpose | Pins |
| --- | --- | --- |
| 9 NeoPixel Buttons | Input buttons with RGB LEDs | Pin 13 |
| White LED Strip | 96 RGB LEDs for ambiance | Pin 53 |
| DY-SV17F Audio | Sound effects player | Serial2 |
| Hall Effect Sensor | Detects handset pickup | Pin 7 |
| Activation Pin | COGS activation input | Pin 28 |
| Completion Pin | COGS win signal output | Pin 30 |
| RGB Ring (External) | Status indicator via UART | Serial2 TX |

### Audio Files Required

| File | Purpose |
| --- | --- |
| /1.wav - /9.wav | Button press tones (DTMF-style) |
| /wrong.wav | Incorrect code sound |
| /winner.wav | Victory fanfare |
| /tone.wav | Dial tone (4:55 duration) |
| /dial.wav | “Please dial” voice prompt |

### Secret Code

// Line 54: The winning pattern
const int pattern[] = { 1, 8, 5, 2, 4 };  // 5-digit code

### Puzzle Flow

┌─────────────────┐
│  STANDBY        │ ← Activation pin HIGH
│  (Inactive)     │   All LEDs off, no audio
└────────┬────────┘
         │ Activation pin goes LOW
         ▼
┌─────────────────┐
│  ACTIVATED      │ ← White strip turns on
│                 │   Plays "Please dial" prompt
│                 │   RGB ring turns GREEN
└────────┬────────┘
         │ After 6 seconds
         ▼
┌─────────────────┐
│  DIAL TONE      │ ← Continuous dial tone plays
│  READY          │   Waiting for button input
└────────┬────────┘
         │ Player presses buttons
         ▼
┌─────────────────┐
│  CODE ENTRY     │ ← Button LEDs light green on press
│  (Rolling       │   Each press plays tone
│   Buffer)       │   Last 5 presses stored
└────────┬────────┘
         │
    ┌────┴────┐
    ▼         ▼
┌───────┐ ┌───────┐
│ WRONG │ │CORRECT│
│ CODE  │ │ CODE  │
│       │ │       │
│wrong  │ │winner │
│.wav   │ │.wav   │
│plays  │ │plays  │
│       │ │       │
│Dial   │ │Rainbow│
│tone   │ │30s    │
│resume │ │       │
└───────┘ └───────┘
              │
              ▼
        ┌─────────────────┐
        │  WIN!           │ ← COGS pin 30 pulses
        │                 │   MCU resets via watchdog
        └─────────────────┘

### Rolling Buffer System

The code uses a “rolling buffer” - only the last 5 button presses matter:

Press sequence: [1] [2] [3] [4] [5] [6] [7]
Buffer after:   [3, 4, 5, 6, 7]  ← Only last 5 kept

To win with code 1,8,5,2,4:
Any presses... then: [1] [8] [5] [2] [4] → WIN!

This means players can keep trying without needing to “clear” - they just need to enter the correct sequence consecutively.

## PAGE 2: What Can Be Tweaked

### ⚡ Quick Settings Reference

| Setting | Location | Default | Purpose |
| --- | --- | --- | --- |
| debounceDelay | Line 61 | 50ms | Button debounce time |
| toneDuration | Line 69 | 4:55 | Dial tone restart interval |
| dialToToneDelayMs | Line 70 | 6000ms | Gap after dial prompt |
| inputTimeoutMs | Line 75 | 10000ms | Inactivity timeout |
| wrongToneExtraDelay | Line 73 | 4000ms | Delay after wrong code |

### 🔢 Changing the Secret Code

// Line 54: Modify pattern array
const int pattern[] = { 1, 8, 5, 2, 4 };  // Current code

// Examples:
const int pattern[] = { 1, 2, 3, 4, 5 };  // Simple sequential
const int pattern[] = { 9, 1, 1 };        // Shorter code (3 digits)
const int pattern[] = { 7, 3, 5, 9, 2, 6, 8 };  // Longer code (7 digits)

Important: Update patternSize is automatically calculated:

const int patternSize = sizeof(pattern) / sizeof(pattern[0]);

### 🔘 Button Pin Assignments

// Lines 22-30: Button input pins
const int button1 = 12;
const int button2 = 3;
const int button3 = 4;
const int button4 = 5;
const int button5 = 6;
const int button6 = 8;
const int button7 = 9;
const int button8 = 10;
const int button9 = 11;

### 💡 LED Settings

Button NeoPixels:

// Lines 10-11: Button LED strip
#define NUM_PIXELS 9
#define NEOPIXEL_PIN 13

// Line 319: Button LED color when pressed
strip.setPixelColor(pixelIndex, strip.Color(0, 150, 0));  // Green

White Ambiance Strip:

// Lines 18-20: White LED strip
#define WHITE_STRIP_PIN 53
#define WHITE_STRIP_PIXELS 96

// Line 142: Activated state brightness
whiteStrip.setPixelColor(i, whiteStrip.Color(255, 255, 255, 70));

### 🔊 Audio Configuration

// Line 124: Volume level
player.setVolume(100);  // 0-100

// Lines 36-52: Sound file paths
const char* wavFiles[] = {
  "/1.wav", "/2.wav", "/3.wav", "/4.wav",
  "/5.wav", "/6.wav", "/7.wav", "/8.wav", "/9.wav"
};
const char* wrongNumberFile = "/wrong.wav";
const char* winnerFile = "/winner.wav";
const char* toneFile = "/tone.wav";
const char* pleaseDialFile = "/dial.wav";

### 🎨 RGB Ring Commands

The external RGB ring is controlled via Serial2:

// sendRingCommand() function - Line 867
'G' = Green (ready state)
'R' = Red (error state)
'O' = Off/Orange

// Usage:
sendRingCommand('G');  // Set ring to green

### ⏱️ Timing Adjustments

Dial Tone Restart:

// Line 69: How long before dial tone restarts
const unsigned long toneDuration = 4 * 60 * 1000 + 55 * 1000;  // 4:55

Delay After Wrong Code:

// Line 73: Pause before dial tone resumes after error
const unsigned long wrongToneExtraDelay = 4000;  // 4 seconds

Inactivity Timeout:

// Lines 75-78: Reset if player stops mid-entry
const unsigned long inputTimeoutMs = 10000;  // 10 seconds
const bool disableInputTimeout = true;  // Currently disabled

### 🧲 Hall Effect Sensor

The sensor on Pin 7 detects when the handset is picked up:

// Line 33: Hall effect sensor pin
const int hallEffectPin = 7;

// When triggered (LOW), it:
// 1. Resets the code buffer
// 2. Plays the welcome/dial prompt
// 3. Reschedules dial tone

### 🎉 Win Celebration

// Lines 391-411: Rainbow animation on win
// Duration: 30 seconds
// Effect: Full strip cycles through rainbow colors

// After celebration:
wdt_enable(WDTO_15MS);  // Watchdog reset
while (true) {}  // Wait for reset

### 🐛 Troubleshooting

| Problem | Solution |
| --- | --- |
| Buttons not registering | Check debounce delay and pin wiring |
| Audio not playing | Verify Serial2 connection and SD card files |
| Code never wins | Check pattern array matches intended code |
| Ring not changing color | Verify Serial2 TX connection to ring |
| Hall sensor always triggering | Check INPUT_PULLUP and sensor polarity |
| Dial tone doesn’t play | Wait 6 seconds after activation |

### 📚 Required Libraries

- Arduino.h - Core functions

- DYPlayerArduino.h - Audio player control

- Adafruit_NeoPixel.h - LED strip control

- Wire.h - I2C (if used)

- avr/wdt.h - Watchdog timer for reset

### 📋 Code Structure

Main Functions:
├── setup()               - Initialize hardware, start activation sequence
├── loop()                - Main game loop, handle state transitions
├── checkButtonState()    - Debounced button reading
├── handleButtonPress()   - Process button input, update buffer
├── setButtonLedState()   - Control button LED on/off
├── playSound()           - Play audio file from SD card
├── sendRingCommand()     - Control external RGB ring
├── anyButtonPressed()    - Check if any button is currently pressed
└── (Watchdog Reset)      - Resets MCU after win

State Variables:
├── isActive              - Puzzle activation state
├── buttonBuffer[]        - Rolling buffer of last 5 presses
├── bufferIndex           - Current buffer position
├── pleaseDialPlayed      - Flag for dial prompt timing
├── toneStartTime         - Dial tone restart tracking
└── inputInProgress       - Mid-entry timeout tracking

### 🔌 Hardware Notes

- NeoPixels: WS2812B (GRB color order) on pin 13

- White Strip: RGB type on pin 53

- Audio: DY-SV17F requires SD card with WAV files

- Hall Sensor: Active LOW with internal pull-up

- Watchdog: Used for reliable MCU reset after win